PLC-logger system description

Customer:	Vistaprint

Foreword:	Description oft he implemented logger system

Project:	Promo V2 printer

Project number:	10116.02.01

Version:	0.1

Filing:	D:\60_Projekte\10116 Vista Print\02 Printer - GEMINI\01 Development\20_Engineering\20_Development\Specification\Logger_System_Description_V0.1.docx

# Contents

1	Contents	2

2	Basics	4

2.1	Introduction	4

2.2	Reference documentation	4

2.3	Version	4

2.4	Abbreviations, definitions, glossary	4

3	Starting position	5

3.1	Short summary of the concept idea	5

4	Best practice	6

4.1	Define/use state log	6

4.1.1	Usage with base controller to write messages	6

4.1.2	File write	6

5	The log system detailed implementation description	7

5.1	General	7

5.2	FB_Queue	7

5.2.1	General	7

5.2.2	System overview	7

5.2.3	Public Methods	8

5.2.4	Constants	9

5.3	FB_BasLog	9

5.3.1	General	9

5.3.2	Structure of a Base-Log entry	10

5.3.3	Public Methods	10

5.3.4	Constants	11

5.4	FB_DatLog	11

5.4.1	General	11

5.4.2	Internals	11

5.4.3	Public Methods	12

5.4.4	Constants	13

5.5	FB_FileLc	13

5.5.1	General	13

5.5.2	System overview	13

5.5.3	Configuration and Template File	14

5.5.4	Public Methods	14

5.5.5	Constants	15

5.6	Other used system functionalities	17

5.6.1	Timestamp	17

5.6.2	Task information	17

# Basics

## Introduction

This document describes the concept and the behaviour of the Vistaprint PLC logger system and is intended for all of the involved software-programmer in the Vistaprint PLC software developing.

## Reference documentation

| Document | Version | Date |
| --- | --- | --- |

## Version

| Description | Author | Status | Version | Date |
| --- | --- | --- | --- | --- |
| basic version | kvo | d | 0.1 | 07.10.2014 |

Status: d = draft, r = released

## Abbreviations, definitions, glossary

| Abbreviation | Designation |
| --- | --- |

# Starting position

The idea is to implement a basic log system which is easy to use and work stand-alone in the PLC software.

The following requirement/features should be possible:

- Configurable over text file or properties

- Extensible for different log application cases

Producer (application task/controller):

- Mutex save (multi-tasking system)

- Independent log entry and file handling (no timing problems or waits)

- Save buffers as retain (never lose any entry even if a system restart occurs)

Consumer/clients

- New file after a time, max number of entries per file or max size of file

- Definable Max count of files in log folder, file stored with a configured period or max total size of all log files in folder

- Option actual log file name stay the same (archive the active log file if it exceeds one of the defined limits to a configurable file name with date and time at the end added)

- Multiple clients for the same log information’s

- Different file formats (csv, xml, html) solved over free format definition or specific clients

- Different client types (e.g. file writer or online watch clients( possible over serial, tcp or other protocols))

Because this system should grow with the application cases the first version will include not all of the features above.

## Short summary of the concept idea

The application task or controller (log writer) subscribes the usage of a logger to the logger system over a name (constant). As soon as a consumer (e.g. csv-file-client) subscribes at the same logger system with the same name a buffer is allocated. The base log system adds a fixed structure to the concerning queue. The application writes over a specific log controller (e.g.  DatLog (log a defined structure) or directly to the base log system). The consumer/client reads out the configured log type information’s from the queues and writes this information well formatted to a file, a bus interface or another implemented interface.

# Best practice

## Define/use state log

The state log entry is already predefined in the BaseCtrl FUB so every controller inherits it from there.

### Usage with base controller to write messages

In the base controller in the FB_INIT the logger entry is switched on per default (“CIf.SwCfg.bolEnStateLogger := TRUE;”). Under me. stStateLogger : T_StateLogger; a new structure is defined.

TYPE T_StateLogger :

STRUCT

pfbBaseLog : POINTER TO FB_BasLog; // should be set from outside over property

uinStatus_Handle : UINT; // =0 handle ok or not allready requested

pHandle : POINTER TO T_Bl_LibHandleRow; // pointer to base logger system

uinStatusWrite : UDINT; // feedback if write was successfully

bolDoLog : BOOL; // state change detected or application cmd to log infos

strText1: STRING(BasLog.cMAX_CHAR_TEXT1); // set in CycleEndDuty

// free usable for application

strText2: STRING(BasLog.cMAX_CHAR_TEXT2); // free usable from application task for additional

debug information’s

dinPar3: DINT;	 // free usable from application task for additional debug information’s

dinPar4: DINT;	 // free usable from application task for additional debug information’s

END_STRUCT

END_TYPE

The access point to the log system has to be defined by initialize the new property “BaseLog” at start up with the address of the central base log instance (pCtrlInst[uinIdx]^.BaseLog := pBasLogFB;). For this a new function called setAllCtrlBasLogFB() to initialize all controllers is implemented. Easier is to use the new initAllCtrl() function than the definition of the alarm manager, log system and the CtrlInit is done in one loop. This function has to be called at start up after all controllers are initialized.

The entries of state/sub state change information are logged automatically in the CycleEndDuty. For the application a string and two values are free to use for additional information. By setting the “me.StateLogger.bolDoLog:=TRUE” a log entry can forced from the application for additional log entries.

Summary need for application automatic log entries:

- MyController.BaseLog := adr(g_fbBasLog) 	(set over global function)

- CIf.SwCfg.bolEnStateLogger := TRUE 	(per default set)

### File write

Copy from … BaseConcept\Document\Logger\StateLogCsv.cfg the example configuration file to a available PLC folder. In the BasePLC sample a FB (FB_FileLc_StateLogger) is already defined with an example of the needed sequence. Copy this FB to your application and change the “In_strCfgFileName” and the “In_strPath” to the before copied configuration file name/folder. Now call the FB at start up to initialize until “Out_bolInitDone”.

//****************************************************************************************************

// Log system - init file client's - takes more than one cycle!

//****************************************************************************************************

// state logger file client: init -> readout config file, allocate memory -> cyclic read out state logs and write to csv file

fbFileLc_StateLogger(In_p_fbBasLog:=ADR(g_fbBasLog),In_p_fbFileLC:=ADR(g_fbFileLc));

IF (fbFileLc_StateLogger.Out_bolInitDone=TRUE) OR  (g_bolLogEnableState = FALSE)  THEN

g_uinStartProzess := g_uinStartProzess + 1; // For StartUp order -> next step!

END_IF

Now add the cyclic call in the cyclic part of a low priority task. Add a failure detection to your application to recognize problems with the logger if needed.

//****************************************************************************************************

// file client call for state logger

//****************************************************************************************************

fbFileLc_StateLogger();

IF (fbFileLc_StateLogger.Out_udiStatus<>0) THEN // state log file client error

;

//fbAlm_Man.setAlarm(p_fbMAIN^.CtrlCIf^.Info.intCtrlId,E_ALM_MAIN.ViperLogFileClient,UDINT_TO_DINT(fbFileClient_ViperLog.Out_udiStatus),ADR(pTmpSIf_MAIN^.Alm),E_AlmState.RST,p_fbMAIN^.CtrlCIf^.Job.eMe_ActivAlmClass);

END_IF

# The log system detailed implementation description

## General

## FB_Queue

### General

The FB_Queue provides methods to create initialize and manage a queue for every consumer and task class. A queue is a FIFO-Buffer (First in First out) with configured length. As basic system functionality the FB_MemRingBuffer from the Beckhoff library Utilities is used.

Beckhoff help: The FB_MemRingBuffer function block allows data records of varying lengths to be written into a ring buffer, or for data records that have previously been written there to be removed from the ring buffer. The buffer operates as a FIFO; data records are read from it in the same sequence in which they were first written into the ring buffer. This means that the oldest entries are the first ones that are read. The buffer storage is provided to the function block via the input variables.

The needed storage buffer size is free definable and is created over the Beckhoff operator __New.

Beckhoff help: The operator __NEW allocates memory for function block instances or arrays of standard data types. The operator returns a suitably typed pointer to the object.

With this system functions is the creation of the queues and information storages very flexible and has only the limit of the available dynamic memory. This system has no retain buffer. This means after a power failure or a restart the last not already saved in file log message could be lost. The great advantage is that in this way only the registered writer or reader takes system resources and only a minimum of static not configurable infrastructure is taken from the PLC.

For tests and as example a FUB FB_QueueTest_Sender and FB_QueueTest_Receiver is also implemented and placed in folder Queue.

### System overview

Receive - Client:
M_Init("q1")
M_GetHandle (“q1“)
M_Read()

Queue System
“q1“

Send-Task PRG_FAST:
M_GetHandle (“q1“)
M_Write()

Send-Task PRG_MIDDLE:
M_GetHandle (“q1“)
M_Write ()

Send-Task PRG_SLOW:
M_GetHandle (“q1“)
M_Write ()

### Public Methods

M_Init

This method should be called one time after start-up of the system. The method allocates an over name defined queue system. Every sender calls the get handle and per caller task class a queue will be allocated automatically.

Interface:

- In_strQueueName: STRING(Queue.cMAX_NAME_LENGTH); // name of the queue system

- In_uinNrOfElement: UINT;		// number of queue elements

- In_udiSizeOfElement: UDINT;	// size of an element

- In_uinMode:UINT;			// 0: no overwrite of buffer, 1: ring buffer overwrite oldest entry

- Out_pHandle: UDINT; 		// handle for every task class sender/receiver

- Out_uinStatus: UINT;		// error -> 0:ok

M_GetHandle

Every sender or receiver of log messages must call this method to get a valid access handle for the required log system. Such a call allocates automatically per task class a queue, if it is not already done before.

Interface:

- In_strQueueName: STRING(Queue.cMAX_NAME_LENGTH); // name of the queue system

- Out_pQueueHandle : POINTER TO BYTE;

M_GetHandle_TC

This method gives back a valid access handle for the required log system and allocates automatically for the forced task class a queue, if it is not already done before. This function is used for internal behaviour of base log.

Interface:

- In_strQueueName: STRING(Queue.cMAX_NAME_LENGTH); // name of the queue system

- In_bytTaskIndex: BYTE; // calling task index

- Out_pQueueHandle : POINTER TO BYTE;

M_Write

Write an element (Variable, Struct, …) based on the before defined M_Init call dimensions.

Interface:

- In_pQueueHandle : POINTER TO BYTE;  // handle from M_GetHandle()

- In_pData: POINTER TO BYTE;  // source address of the write element

M_Read

Reads and delete the oldest element from the queue.

Interface:

- In_pQueueHandle : POINTER TO BYTE;  // handle from M_GetHandle()

- In_pData: POINTER TO BYTE; // destination address of the read element

M_ReadReference

This method reads the oldest element from the queue.

Interface:

- In_pQueueHandle : POINTER TO BYTE;  // handle from M_GetHandle()

- Out_pData: POINTER TO BYTE;  // address of the read element

M_Clear

Clear specific queue system.

Interface:

- In_pQueueHandle : POINTER TO BYTE;  // handle from M_GetHandle()

### Constants

Error numbers

cERR_QUEUE_FULL : UINT := 1;

cERR_QUEUE_EMPTY : UINT := 2;

cERR_QUEUE_NAME : UINT := 50000; // not valid or empty queue name

cERR_NR_OF_ELEMENTS : UINT := 50001; // number of elements =0

cERR_SIZE_OF_ELEMENTS: UINT := 50002; // element size =0

cERR_MEM_ALLOC  : UINT := 50003; // error allocate memory

cERR_QUEUE_DOES_NOT_EXIST : UINT := 50004; // queue does not exist

cERR_Q_INFO_DOES_NOT_EXIST : UINT := 50005; // queue info does not exist

cERR_DATA_INVALID : UINT := 50006; // pointer to data not valid

cERR_HANDLE_INVALID : UINT := 50007; // handle not ok

cERR_TC_INVALID : UINT := 50008; // task class index not ok

cERR_CLEAR : UINT := 50009; // clear of queue not ok

Limits

cMAX_NAME_LENGTH : UINT := 80; // maximal string length of queue name

cMAX_NR_OF_TASKS: USINT := 6; // for the moment enough

## FB_BasLog

### General

The BasLog provides the basic log functionality based on Queue system for the further special loggers. The available methods allow making standardized entries in the queue and reading out from there. Every entry has a set of parameters and gets a time stamp. For the time stamp is the basic FB_TimeStamp used. So the user can define if he likes time stamps in UTC or local windows time. At start-up of the system there are two method calls required. To allocate the required queues for the data writer/read call M_ClientInit(). Over the call M_GetHandle() you get the required handle for writing and reading to the log system. It is possible to manage several clients. The system manages the entry itself based on the defined mask. The bit coded mask allows filtering the entries. If an entry mask does not match any client mask the entry will be rejected. The queue entry is done based on calling task information. So it is important that the received handle from the call of the M_GetHandle() is only used in the same task.

For tests and as example a FUB FB_BasLogTest_Sender and FB_BasLogTest_Client is also implemented and placed in folder Queue.

### Structure of a Base-Log entry

| Parameter | Data Type | Wert |
| --- | --- | --- |
| FB_QueueTest_Receiver | DATE_AND_TIME | Actual date and time (UTC or local time) |
| uinMs | UINT | Actual milliseconds |
| udiLogType | UDINT | Typ of log entry (bit coded) |
| intCtrlID | INT | ID of the controller, which has the entry added |
| strText1 | STRING[AE_BL_MAX_CHAR_TEXT1] | Information text 1 (free choice) |
| strText2 | STRING[AE_BL_MAX_CHAR_TEXT2] | Information text 2 (free choice) |
| dinPar1 | DINT | Parameter 1 (free choice, 4 byte max.) |
| dinPar2 | DINT | Parameter 2 (free choice, 4 byte max.) |
| dinPar3 | DINT | Parameter 3 (free choice, 4 byte max.) |
| dinPar4 | DINT | Parameter 4 (free choice, 4 byte max.) |

The Parameters 1 to 4 are declared as DINT, could be used for every data type with max size of 4 Byte. The different types have to be checked in the readout method!

### Public Methods

M_GetHandle

Every sender or receiver of log messages must call this method to get a valid access handle to the log system. Such a call allocates automatically per task class a queue, if it is not already done before.

Interface:

Out_pHandle : POINTER TO T_Bl_LibHandleRow; // handle of log system for write (M_Log)

Feedback =  0 => successful done, <>0 => error

M_Log

Add base log with actual time stamp to log system.

Interface:

- In_pHandle : POINTER TO BYTE; // handle of log system

- In_udiLogType: UDINT; // log type

- In_intCtrlID: INT; // unique controller ID which has created this entry

- In_strText1: STRING(BasLog.cMAX_CHAR_TEXT1);	// information text1

- In_strText2: STRING(BasLog.cMAX_CHAR_TEXT2); // information text2

- In_dinPar1: DINT; // value parameter 1

- In_dinPar2: DINT; // value parameter 2

- In_dinPar3: DINT; // value parameter 3

- In_dinPar4: DINT; // value parameter 4

Feedback =  0 => successful done, <>0 => error

M_ClientInit

This method should be called one time after start-up of the system for reading client. The method allocates an over name defined queue system.

Interface:

- In_strClientName: STRING(Queue.cMAX_NAME_LENGTH); // name of the client

- In_udiMask: UDINT;			// filter mask which log type should be logged for this client

- In_uinNrOfElement: UINT;			// number of elements in queue

- In_p_fbTimeStamp: POINTER TO FB_TimeStamp;

- In_bolUseLocalTimeStamp : BOOL;		 // use local time stamp or UTC

- Out_pHandle: POINTER TO T_Queue_Info; 	// handle for read

- Out_uinStatus: UINT; 	// feedback value 0:ok, <>0 => error

M_Read

Read and remove oldest base log with defined mask from the log system.

Interface:

- In_pHandle : POINTER TO POINTER TO BYTE; // handle for read

- In_pData: POINTER TO T_Bl_BaseStruc; // Pointer for feedback element

Feedback =  0 => successful done, <>0 => error

M_ChangeMask

This method changes the log mask of a client.

Interface:

- In_strClientName: STRING(Queue.cMAX_NAME_LENGTH); // name of the client

- In_udiNewMask: UDINT;		// new filter mask which log type should be logged for this client

Feedback =  0 => successful done, <>0 => error

### Constants

Error numbers

cERR_TOO_MANY_CLIENTS : UINT := 51001; // max number of clients connected

cERR_SYSTEM_NOT_FOUND : UINT := 51002; // the system with this name does not exist

cERR_INVALID_LOGTYPE : UINT := 51003; // log type is not correct bit coded

cERR_NO_WRITE_TASK : UINT := 51004; // no client added (no handle from a task)

cERR_NO_HANDLE : UINT := 51005; // no handle received => fbGetHandle()

cERR_NO_FB_TIME_STAMP_POINTER : UINT := 51006; // no pointer for FB_TimeStamp

cERR_NO_FB_TIME_STAMP_INIT_NOT_OK : UINT := 51007; // time stamp value not ready for read out

Log types

cLOGTYPE_ALARM : UDINT:=1; //logtyp alarm

cLOGTYPE_WARNING : UDINT:=2; //logtyp warning

cLOGTYPE_INFO : UDINT:=4; //logtyp info

cLOGTYPE_STATELOG : UDINT:=8; //logtyp state log (for statmachine diagnostic)

cLOGTYPE_PV_EVENT : UDINT:=16; //logtyp triggered value log

cLOGTYPE_PV_CYCLIC : UDINT:=32; //logtyp cyclic value log

cLOGTYPE_DATA : UDINT:=64; //logtyp data logger

Limits

cMAX_CLIENTS : USINT := 5; 	// maximal number of clients

cMAX_CHAR_TEXT1: USINT := 80;	// maximal number of chars for information text1

cMAX_CHAR_TEXT2: USINT := 80;	// maximal number of chars for information text2

## FB_DatLog

### General

The DatLog FB can be used for logging application defined complex data structures. The on this way logged information’s could also be read out from a client (i.e. FileCl). The advantage to the base log system is that the data points will be type casted and the number of structure elements is not fixed. The data log system has to be initialized one time (M_Init). For the logging task the method M_GetHandle() has to be called one time.

### Internals

The single data point of the given strut is sent as one base log message. To be sure that one set is together a start and stop record control message is sent.

Example of a data record

Initialized with start Ident 100!

stMesswerte.usint1 := 33;

stMesswerte.uint1 := 34;

stMesswerte.udint1 := 35;

stMesswerte.usint2 := 36;

stMesswerte.rea1 := 36.5453;

stMesswerte.dt1 := DT#1973-02-12-04:33;

### Public Methods

M_GetHandle

Every sender or receiver of log messages must call this method to get a valid access handle to the log system. Such a call allocates automatically per task class a queue, if it is not already done before.

Interface:

- In_strDatLogName: STRING(DatLog.cMAX_SYS_NAME_LENGTH); // name of the client

- In_p_fbBasLog: POINTER TO FB_BasLog; // only one central FB_BasLog instance allowed per PLC!!!

- Out_pHandle : POINTER TO BYTE; // handle of log system for write (M_Log)

- Out_udiStatus : UDINT; //0 => successful done, <>0 => error

M_Log

Add dat log with actual time stamp to log system.

Interface:

- In_pHandle : POINTER TO BYTE; // handle of log system for write (M_Log)

- In_a_pData: ARRAY [0..DatLog.cMAX_NR_OF_VAR] OF POINTER TO BYTE; // address of structure elements

- Out_udiStatus: UDINT; // 0 => successful done, <>0 => error

M_Init

This method should be called one time after start-up of the system for reading client. The method allocates an over name defined log system.

Interface:

- In_bolExecute : BOOL;

- In_strDatLogName: STRING(DatLog.cMAX_SYS_NAME_LENGTH); // name of the client

- In_a_strDataDefinition: ARRAY[0..DatLog.cMAX_NR_OF_VAR] OF STRING(DatLog.cMAX_VAR_NAME_LEN); // complete VarName: p.e.:MAIN.INIT_TASK.VARINT

- In_uinStartId: UINT;	// Start ident for item index -> cfg file config has to match with this number!

- In_Port : T_AmsPort;	// on plc the first runtime system is the 801, 811 ...

- Out_udiStatus: UDINT;		 // 0 => successful done, <>0 => error

- Out_bolDone: BOOL;		// Done

### Constants

Error numbers

cERR_MEM_ALLOC : UINT := 53001; // error on allocating memory

cERR_NO_HANDLE : UINT := 53002; // no handle received => fbGetHandle()

cERR_NO_DATE : UINT := 53003; // no valid data pointer received

cERR_INIT_FAILED : UINT := 53006; //

cERR_NO_ITEMS : UINT := 53004; //

cERR_UNSUPPORTED_TYPE : UINT := 53005; // the system support this datatyp not

cERR_UNSUPPORTED_TYPE_STRING_TOO_LONG : UINT := 53006; // defined string in base log shorter then log string

Limits

cMAX_SYS_NAME_LENGTH : USINT := 30; //max name length of system data log name

cMAX_VAR_NAME_LEN : USINT := 50; // max len of variable name

cMAX_NR_OF_VAR : UINT := 20; // max number of variables definable to log

## FB_FileLc

### General

The file client function block subscribes itself in the base log system to read out configured log messages. He interprets the messages and writes it in the template part of the configuration file defined format to a file. The file name, extension, write cycle and the all other necessary parameters are defined in a configuration file. So the user can free define if he wish to have a *.csv, *.html or xml file.

The file client can at the moment the following standard log types interpret:

- Base - Log Einträge (For State-, Debug- and/or Tracelogs)

- Data - Log Records (FB_DatLog)

- Open - Log Records (User defined records)

The records are data sets of base log messages with a start and end tag marked. For details take a look in the chapter oft he data log.

To be sure that only one client initialization is active at the same time and a unique client name is used, a central file client structure is required. It is recommended the use only one instance per application. The writing and reading of files need a lot of ressources so ist important to call the file client in a fast as possibel task with low priority and no requirement for real time.

### System overview

PV-Log: [Var1, Var2, Var3, Var4]

FileClient
FB_FileLC.M_Init ()
FB_FileLC.M_Cyclic ()

BaseLogSystem

Base-Log: [LogType, CtrlId, String1, String2, Par1..4]

Data-Log: [value1, value2, value3]

The FileClient notify itself in  BaseLog system as client.

Configuratio/template file:
<CONFIG>
</CONFIG>

<TEMPLATE>
</TEMPLATE>

LogFiles

### Configuration and Template File

In this file are in the first part the parameters of the file client defined:

In the second part of the file the log output file format and value order is defined:

Under the logger information folder some examples should be available. Behind every parameter a comment should make clear the possible values and the behaviour.

### Public Methods

M_Init

Initialization of file client based on the information read out from the configuration file. Because of the usage of the Beckhoff file io FUB’s it needs some PLC task cycles to read out. A cyclic call of the M_Init method is required until Out_udiStatus<>0 or Out_bolDone=TRUE. The M_Init() method subscribes the client as log message reader to the log system.

Interface:

- In_strCfgFileName: STRING(FileLc.cFILENAME_LENGTH); // name of the config/template file

- In_strPath: STRING(FileLc.cPATH_LENGTH);  // path of the config/template file

- In_p_fbBasLog: POINTER TO FB_BasLog; // only one central FB_BasLog instance allowed per PLC!!!

- In_bolExecute : BOOL;

- In_p_uinFLcIdx : POINTER TO UINT; // actual file client id

- Out_pHandle: POINTER TO T_FileLc_SysInfo; 	// handle for cyclic call

- Out_udiStatus: UDINT; 	// 0 => successful done, <>0 => error

- Out_bolDone: BOOL;	// Done

M_Cyclic

The Cyclic call of the file client writes in required time periods or if buffer is full the configured log information to the log file. Sub function blocks for file handling (FB_FileHdl) and folder supervision (FB_FolderHdl) are also called in this cycle.

Interface:

- In_pHandle: POINTER TO T_FileLc_SysInfo; // handle for cyclic call

### Constants

Error numbers

cERR_NBR_OF_FC_EXEEDED : UINT := 50400; (*Max number of file clients reached*)

cERR_FILENAME_MISSING : UINT := 50401; (*Filename missing*)

cERR_PATH_MISSING : UINT := 50402; (*Path definition missing*)

cERR_CONFIG_TAG : UINT := 50403; (*Error in <CONFIG></CONFIG> tag*)

cERR_TEMPLATE_TAG : UINT := 50404; (*Error in <TEMPLATE></TEMPLATE> tag*)

cERR_INVALID_LOGTYP : UINT := 50405; (*Unknown or missing log type -> template/config - file*)

cERR_HEADER_TAG : UINT := 50406; (*Error in <HEADER></HEADER> tag*)

cERR_RECORD_TAG : UINT := 50407; (*Error in <RECORD></RECORD> tag*)

cERR_ROW_TAG : UINT := 50408; (*Error in <ROW></ROW> tag*)

cERR_ITEM_TAG : UINT := 50409; (*Error in <ITEM></ITEM> tag*)

cERR_FOOTER_TAG : UINT := 50410; (*Error in <FOOTER></FOOTER> tag*)

cERR_HANDLE : UINT := 50411; (*no handle reached*)

cERR_INVALID_DEVICE : UINT := 50412; (*File destination not correct*)

cERR_ALLOC_BUFFER_FILE_CONFIG : UINT := 50413; (*No memory available for config read out*)

cERR_ALLOC_MEMORY : UINT := 50414; (*No memory available*)

cERR_NO_BASELOG : UINT := 50415; (*BaseLog pointer invalid*)

cERR_NO_FLCIDX : UINT := 50416; (*INIT not possibel no valid act client nr index reference / pointer invalid*)

Limits

cPRE_POST_TEXTLEN : UINT := 256; // max text len for pre and post text of a value

cMAX_RECORDS_IN_FILE_BUF : UINT := 4; (*factor for calculation of file buffer*)

cMAX_NBR_OF_FILECLIENTS : UINT := 10; (*Max number of file clients*)

cFILENAME_LENGTH : UINT := 260; // config file name len

cPATH_LENGTH : UINT := 260; // folderpath string len

Attention for using UDINT data types! At the moment only the DINT range is supported!

Config/Template

//<ITEM>***</ITEM> predefined items of base log

cITEM_UNIX_DATE_TIME_STR : STRING[80] := 'DT_UNIX_STR'; (*Item UNIX Datum/Zeit als String*)

cITEM_UNIX_DATE_TIME : STRING[80] := 'DT_UNIX'; (*Item UNIX Datum/Zeit als Integer*)

cITEM_TIME_MILLI : STRING[80] := 'TIME_MILLI'; (*Item Millisekunden*)

cITEM_EXCEL_DATE_TIME : STRING[80] := 'DT_EXCEL' (*Item date/time in Excel format. Days as real since

1.1.1970*)

cITEM_DATE_SLASH_TIME_COLON : STRING[80] := 'DATE_SLASH_TIME_COLON'; //english date format 2014/09/23 07:10:24

cITEM_BL_LOGTYP : STRING[80] := 'BL_LOGTYP'; (*Base Log Item LogTyp*)

cITEM_BL_CTRL_ID : STRING[80] := 'BL_CTRL_ID'; (*Base Log Item Controller Id*)

cITEM_BL_STRING1 : STRING[80] := 'BL_STRING1'; (*Base Log Item STRING 1*)

cITEM_BL_STRING2 : STRING[80] := 'BL_STRING2'; (*Base Log Item STRING 2*)

cITEM_BL_PAR1 : STRING[80] := 'BL_PAR1'; (*Base Log Item PAR1  *)

cITEM_BL_PAR2 : STRING[80] := 'BL_PAR2'; (*Base Log Item PAR2*)

cITEM_BL_PAR3 : STRING[80] := 'BL_PAR3'; (*Base Log Item PAR3*)

cITEM_BL_PAR4 : STRING[80] := 'BL_PAR4'; (*Base Log Item PAR4*)

// <LOG_TYPE>****</LOG_TYPE>	 predefined values

cLOGTYP_BL_STR : STRING[80] := 'BL'; (*BaseLog*)

cLOGTYP_PV_STR : STRING[80] := 'PV'; (*PVLog*)

cLOGTYP_DATA_STR : STRING[80] := 'DATA'; (*DataLog*)

cLOGTYP_OPEN_STR : STRING[80] := 'OPEN'; (*General  (Attention-> Mask has to be defined)*)

## Other used system functionalities

### Timestamp

In the log system the FB_TimeStamp (FB_TimeStamp_Description_V0.1.docx) is used. Because the local windows time is not immediately and exact accessible at every PLC cycle.

### Task information

The function GETCURTASKINDEXEX finds the task index of the task from which it is called.

GETCURTASKINDEXEX : Returns the current task index of the calling task. The following return value are possible:
-1 : Function is called by Windows context.
0 : Function is called by realtime context but not from a cyclic PLC task.
1 bis n : Function is called by a cyclic PLC task. The return value is the task index.